# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Procurement of equipment for digitization of the pathology laboratory

Ref. number: 1568-24-DigiPath-05

Columns 1-2 should be completed by the Project partner

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Project partner shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

The requirements set out in the technical specifications represent the minimum technical characteristics which offered goods must satisfy, unless stated otherwise, and tenderers are not allowed to modify technical specification in any way.

For each item for which it is not explicitly stated that it is allowed to offer goods of the equal characteristics, i.e. for each item where it is not stated “or equivalent”, for the purposes of this tender documentation it is assumed that words “or equivalent” are stated, and tenderer is allowed to offer equivalent goods / goods of equivalent characteristics.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s decision (Y/N) |
| --- | --- | --- | --- | --- |
| 1 | Item name: Microscope Quantity: 1 piece Minimum requirements: - Brightfield illumination feature with adjustable light intensity on the stand with intensity indicators - The system must include halogen light (minimum 25W) and LED light (minimum 3W), with the ability for the user to interchange between lights - Objective holder with at least 5 positions to accommodate multiple objectives - Plan-Achromat objectives with the following specifications: 4x/0.10, 10x/0.25, 20x/0.45, 40x/0.65,100x/1.20 - 10x eyepieces with a minimum field of view of 22mm or equivalent - Pupillary distance adjustment should cover a range of minimum 48mm to 75 mm - Abbe condenser with a numerical aperture of at least 0.9/1.25 - The mechanical stage must have minimum dimensions of 180x135 mm with a movement range of at least 75x50mm - The system should include at least two slide holder positions for easy slide change - Yellow, green and blue filters for optimal contrast and illumination control - Ability to memorize light intensity settings for each objective - Binocular phototube with: - Ergonomic inclination not exceeding 25° - Adjustable eyepieces - Field of view of at least 20mm |  |  |  |
| 2. | Item name: Automated microtom Quantity: 1 piece Minimum requirements: - Fully Automated functionalities for cutting, trimming, and sample alignment - Must support dual-side operation capability or similar flexibility for user convenience - Touchscreen LCD user interface or other intuitive control options may be acceptable - Wireless remote control for cutting operations - Adjustable cutting thickness range of at least 1 to 100 microns - Adjustable trimming thickness minimum range of 1 micron and a maximum range of at least 200 microns - Adjustable cutting and trimming speed with a minimum of 10 mm/sec to at least 400 mm/sec - Adjustable return speed with a minimum of 10 mm/sec to at least 400 mm/sec - Edge alignment precision of ±0.05° minimum - Automatic sample positioning control for precision cutting - Minimum 10 customizable cutting programs, with the flexibility to adjust slice thickness and cutting speed |  |  |  |
| 3. 4. | Item name: Digital Scanner Quantity: 1 piece Minimum requirements: - Capacity of minimum 1 to maximum 5 slides of standard dimensions (75.0–76.2 x 25.0–26.0 mm) - Capability to scan larger slides with dimensions of 75.0–76.2 x 50.0–51.0 mm is preferred - scanning throughput of minimum 25 slides/hour - Must support dual objectives, with magnifications of 20x and 40x or similar - Continuous zoom with magnification options minimum 80x, including incremental steps - Multi-Layer & Extended Focus capability or equivalent features - Analyses tools for pattern recognition, cell quantification, and histological quantification - Image analysis platform designed as a decision support system for rapid, field-of-view-based, automated quantification for Progesterone, Estrogen, Ki67, Her2 - Software must provide options for case assignment, slide management, and forwarding through a digital scanner interface - Integrated or compatible barcode scanning capability is required - A server with a minimum storage capacity 20TB for slide storage and management - The scanner should support teleconsultation capabilities and include a DICOM format license for digital slide handling - The system should be capable of supporting at least 5 concurrent users working simultaneously without degradation in performance or functionality Item name: Manual Tissue Microarrayer Quantity: 1 piece Minimum requirements: - For extraction from one or more donor tissue blocks - Each individual tissue within the array can be examined under the microscope - Portable - Appropriate for steel array block and cassette array block |  |  |  |
| 5. | Training and after sale services -Testing and training for final users should be privide within 15 calendar days starting from the day of finished installation and commissioning of equipment. -After-sales service over 1 year |  |  |  |